BATTERYGUARDIAN AI

Hardware Specifications, Firmware Analysis & End-to-End System Architecture

Hardware Prototype: 3S1P IMR-18650 Li-ion (11.1V / 1200mAh) + ESP32 Node Discharge Load: 100Ω 50W Wirewound Resistor (C/10 Controlled Discharge, 1.59W max) Sensors & ADC: 100kΩ/33kΩ Dividers (0.2481 ratio) + 100nF RC Filter, INA219, DS18B20 1-Wire Software Architecture: ESP32 BLE GATT → Flutter (SQLite) → FastAPI REST → Public-Dataset ML (NASA PCoE)

1. BatteryGuardian Hardware Baseline & Specifications

The physical prototype is engineered specifically for repeatable, stable lithium-ion degradation telemetry without thermal drift or ADC saturation:

| Subsystem / Parameter | Physical Specification | Design Rationale & Impact on ML |
| --- | --- | --- |
| Battery Pack Configuration | 3S1P IMR-18650 Li-ion Cells Nominal: 11.1V (3x3.7V), 1200 mAh (13.32 Wh) Full Charge: 12.6V (4.2V/cell) Discharge Cutoff: 9.0V (3.0V/cell) | High-current Li-ion chemistry with a linear discharge slope suitable for capacity tracking and SoH estimation. |
| Discharge Load Resistor | 100Ω, 50W Aluminum-Housed Wirewound Current Draw: 126 mA (12.6V) to 90 mA (9.0V) Power Dissipation: 1.587W max Discharge Time: ~9.5 – 10.0 Hours | Running a 1.59W load on a 50W rated heatsink yields a >30x safety margin. Thermal drift is virtually zero, ensuring exceptionally pure voltage curves for ML feature extraction. |
| Voltage Divider Network | Top Resistor: 100kΩ Bottom Resistor: 33kΩ Divider Ratio: 33 / 133 = 0.24812 Max Input Voltage: 12.6V x 0.24812 = 3.126V | Pins 34 (Tap 1), 35 (Tap 2), 32 (Tap 3). Scaled perfectly below the 3.3V ESP32 ADC maximum limit with high impedance to minimize quiescent drain. |
| Hardware RC Low-Pass Filter | 100nF Ceramic Capacitor in parallel with each 33kΩ bottom resistor Cutoff Frequency: fc ≈ 48.2 Hz | Hardware-level anti-aliasing filter attenuates high-frequency switching noise and ADC sampling glitches before digital processing. |
| Temperature & Current Sensors | DS18B20 1-Wire Digital Probes (GPIO 4 with 4.2kΩ/4.7kΩ pull-up) INA219 I2C High-Side Current Sensor (GPIO 21/22, 0.1Ω Shunt, ±3.2A) | Multi-point cell surface temperature tracking without analog ADC drift; bidirectional current monitoring with 0.8mA resolution. |

2. Firmware Code Inspection & Audit

A detailed analysis of the existing Stage 11 test firmware reveals strong foundations alongside key areas requiring upgrade for startup-level production:

• Working Pin Configurations: GPIO 21 (SDA) & 22 (SCL) for I2C (OLED & INA219); GPIO 4 for DS18B20 1-Wire; GPIO 34, 35, 32 for differential cell ADC sensing.

• Divider Constant Alignment: The legacy code used R_TOP=33k / R_BOTTOM=10k (0.2325). The firmware must be calibrated to the actual hardware: R_TOP=100k / R_BOTTOM=33k (Ratio = 0.24812).

• Differential Node Voltage Math: Cell 1 = V_node1; Cell 2 = V_node2 - V_node1; Cell 3 = V_node3 - V_node2. Correctly handles the 3S series voltage ladder.

• Blocking Delay Elimination: The legacy loop used a blocking delay(2000). For reliable BLE transmission and responsive OLED UI, this will be replaced with non-blocking millis() task scheduling.

• BLE GATT Server Integration: The existing test code lacks BLE. We will inject the BLE GATT Server stack with periodic JSON notifications (<180 bytes) on characteristic beb5483e-36e1-4688-b7f5-ea07361b26a8.

3. Real-Time Telemetry Protocol Specification

Structured JSON telemetry payload emitted by the ESP32 every 1000ms:

{ "ts": 1725201600, "c1": 4.120, "c2": 4.085, "c3": 4.032, "v_pack": 12.237, "curr": 0.122, "t1": 28.5, "t2": 28.7, "t3": 29.1, "imb": 0.088, "state": "DISCHARGE", "cycle": 1 }

4. Public Battery Dataset Strategy (NASA Ames PCoE)

To ensure scientific validity without fabricating artificial degradation data, ML models (SoH Regressor & RUL Estimator) are trained on the benchmark NASA Ames Prognostics Center of Excellence (PCoE) Li-ion aging dataset (B. Saha & K. Goebel, 2007).

• True State of Health (SoH):
  SoH_k = (Measured Discharge Capacity_k / Nominal Rated Capacity [1.20 Ah]) × 100%
• True Remaining Useful Life (RUL):
  RUL_k = Cycle_EOL - Cycle_k (where EOL is reached at 80% capacity retention, i.e., 0.96 Ah).

5. Milestone 1 Deliverables & Verification

• Modular Firmware Scaffolding: Scaffold firmware/esp32/ with modular files: config.h, sensors.h, ble_service.h, oled_ui.h, main.cpp.

• 100k/33k Divider & RC Filtering: Implement precise 0.24812 divider ratio, 16-sample rolling average, and ESP32 ADC characterization curve.

• DS18B20 Multi-Probe Driver: Robust 1-Wire scan and non-blocking asynchronous temperature reading for up to 3 individual cell probes.

• INA219 Current & Energy Integrator: Continuous current monitoring and trapezoidal numerical integration for Ah/Wh tracking during discharge.

• SSD1306 Real-Time OLED Dashboard: Structured 4-row layout: Pack V, Current, C1/C2/C3 voltages, temperatures, and BLE connection status icon.

• BLE GATT Server & Notifications: Custom Service UUID 4fafc201... with Telemetry Notify Characteristic beb5483e... broadcasting valid JSON every 1000ms.

• Unit Tests & Hardware Documentation: Complete docs/hardware.md with wiring schematics and automated Python simulation tests verifying parser integrity.